### Doc_02_WiFi_Devices_Part2.docx

#### Section 1: Hardware Staging & Setup (Policies 101–120)

- Pre-Configuration Staging: Enterprise Wi-Fi gateways undergo MAC address pre-registration before dispatch to customer premises.

- PoE Power Requirements: Power over Ethernet (PoE) access points must be powered using 802.3at or 802.3bt compliant switches.

- Outdoor Unit Weatherproofing: Outdoor rated Wi-Fi units must be installed using included weatherproof cable glands and sealants.

- Antenna Positioning Standard: Adjustable dipole antennas should be angled vertically at 90 degrees for optimal horizontal coverage.

- Guest Network Isolation: Frontline staff must instruct users that Guest Wi-Fi networks must remain isolated from primary LAN networks.

- VLAN Segmentation Rules: Managed Wi-Fi access points support up to 8 distinct VLAN tags per physical radio interface.

- Static IP Configuration: Custom static IP assignments must fall outside the default DHCP scope range to avoid IP conflicts.

- Loop Detection Protocol: Enabling Spanning Tree Protocol (STP) on mesh backhaul switches is required to prevent network loop crashes.

- Maximum Client Density: Standard residential nodes support up to 64 concurrent active client connections per band.

- SSID Naming Constraints: Network names (SSIDs) must not exceed 32 UTF-8 characters and should avoid special symbols.

- WPA3 Security Mandatory Mode: Wi-Fi 6E and Wi-Fi 7 radios must operate with WPA3-SAE encryption enabled by default.

- Band Steering Logic: Dual-band routers automatically steer capable dual-band clients to 5GHz or 6GHz bands based on signal strength.

- DFS Channel Radar Notice: Channels operating on Dynamic Frequency Selection (DFS) frequencies may take 60 seconds to initiate.

- Transmission Power Adjustments: Transmit power should be configured to 'Auto' unless manual site survey adjustments are required.

- Channel Width Selection: 160MHz and 320MHz channel widths should only be deployed in low-density wireless environments.

- Beacon Interval Default: Standard beacon intervals are fixed at 100ms unless specialized low-power IoT optimizations apply.

- MU-MIMO Activation: Multi-User MIMO feature sets must remain active across 5GHz and 6GHz bands for multi-device efficiency.

- Explicit Beamforming: Explicit Beamforming technology is enabled natively to improve directional signal to mobile clients.

- DNS Redirection Safeguard: Custom DNS entries configured on routers revert to secure ISP default DNS if upstream fails.

- Hardware Factory Reset Procedure: Holding the rear recessed reset button for 10 consecutive seconds restores factory defaults.

#### Section 2: Technical Troubleshooting (Policies 121–140)

- Diagnostic LED Codes: Solid red WAN LED indicates loss of physical link; flashing yellow indicates DHCP lease negotiation failure.

- Overheating Safety Shutdown: Hardware automatically throttles transmit power or shuts down if internal CPU heat exceeds 85°C.

- Ping & Latency Thresholds: Local gateway ping times exceeding 15ms over wired connections indicate local hardware degradation.

- Packet Loss SLA: Local wired network packet loss must remain under 0.5% during normal operating network load.

- Radio Interference Mitigation: Frontline staff should suggest moving 2.4GHz routers away from active microwave ovens and cordless phones.

- DHCP Scope Exhaustion: If client devices display 169.254.x.x IP addresses, staff must expand the router DHCP address pool range.

- Double NAT Elimination: Customer modems with built-in routers must be placed in Bridge Mode when connected to new Wi-Fi routers.

- MTU Size Standard: Default WAN Maximum Transmission Unit (MTU) size is fixed at 1500 bytes for standard fiber connections.

- PPPoE MTU Adjustment: PPPoE DSL/Fiber configurations require setting the WAN MTU to 1492 bytes to avoid packet fragmentation.

- Port Forwarding Security: Opening WAN ports via port forwarding requires assigning fixed static local IPs to destination hosts.

- UPnP Safety Warning: Universal Plug and Play (UPnP) is disabled by default to mitigate potential IoT vulnerability vectors.

- Remote Management Access: Remote HTTPS administrative WAN access is disabled unless explicitly enabled with a security key.

- Log Export Requirement: Customer support troubleshooting tickets require attaching system syslog exports from the gateway.

- Packet Capture Diagnostic: Advanced B2B support engineers can execute 60-second pcap traces via cloud portal tools.

- Signal Strength Target: Client signal levels should maintain -67 dBm or stronger for seamless voice and video call roaming.

- Roaming Threshold (802.11k/v): Fast roaming kicks in when client RSSI drops below -70 dBm, pushing handover to closer nodes.

- Bandwidth Throttling Check: Quality of Service (QoS) rules must be reviewed if clients report unexpected upload speed caps.

- IPv6 Native Support: Dual-stack IPv4/IPv6 operation is supported natively using DHCPv6 Prefix Delegation.

- WAN Auto-Sensing: Multi-Gig WAN/LAN ports automatically negotiate link speeds between 100Mbps, 1Gbps, 2.5Gbps, and 10Gbps.

- Hardware Cold Reboots: Cold reboots require unplugging power cables for 30 full seconds before reconnecting.

#### Section 3: Rental & Subscriptions (Policies 141–160)

- Rental Device Ownership: Hardware provided on rental plans remains the exclusive property of the service provider at all times.

- Non-Return Fee Assessment: Unreturned rental hardware following account cancellation results in an automatic Unreturned Equipment Fee.

- Equipment Replacement Limits: Rental plans cover up to 2 free functional equipment replacements per 24-month service period.

- Security Feature Subscription: Advanced malware blocking and URL filtering services require an active Security Subscription.

- Parental Control Profiles: System supports creating up to 16 distinct user profiles with tailored schedule blocks and content filters.

- Subscription Auto-Renewal: Monthly add-on subscriptions automatically renew on billing cycle dates unless canceled 48 hours prior.

- Rental Transfer Restrictions: Rental hardware cannot be transferred, sold, or moved to unassigned customer account locations.

- Accidental Loss Insurance: Optional $2/month Equipment Protection covers liquid spill and physical drop damage on rental units.

- Hardware Rental Tier Swap: Customers can swap rental hardware tiers at any time; billing adjusts on the next billing statement.

- Trial Period Cancellation: Add-on software subscriptions feature a 14-day free trial; canceling within trial avoids charges.

- Firmware Feature Lock: Advanced features (e.g., VPN Server) may be locked if hardware rental account payments fall overdue.

- Bulk Business Rentals: B2B rental contracts over 20 nodes include SLA guarantees for same-day replacement dispatch.

- Legacy Rental Sunsetting: Older rental models will be replaced proactively by the company at no charge during scheduled maintenance.

- Subscription Pause Rights: Customers can temporarily pause software subscriptions for up to 90 days during seasonal travel.

- Mesh Node Rental Add-On: Additional rental mesh satellites can be added to existing accounts for $5 per month per unit.

- Self-Install Kit Delivery: Self-installation rental kits are dispatched via standard ground courier within 2 business days of order.

- Professional Install Option: Customers can elect professional technician deployment of rental hardware for a flat $50 charge.

- Returned Rental Cleaning: Returned rental units undergo commercial sanitization, housing replacement, and factory re-flashing.

- Software Feature Licensing: Premium router software add-ons are licensed per account, supporting multiple associated gateways.

- Contract Rollover Status: Upon completing a 24-month rental contract, plans continue month-to-month with free upgrade eligibility.

#### Section 4: Security & Privacy Rules (Policies 161–180)

- Zero-Knowledge Password Storage: Customer Wi-Fi passwords stored in cloud management tools are hashed using AES-256 encryption.

- Telemetry Data Opt-Out: Customers may opt out of anonymous hardware telemetry data collection through the mobile app settings.

- Automatic Vulnerability Patching: High-severity zero-day security vulnerabilities trigger emergency auto-updates regardless of settings.

- Admin Panel Brute-Force Lock: 5 consecutive failed administrative login attempts lock management access for 15 minutes.

- DNS-over-HTTPS (DoH): Routers support DoH to encrypt outbound DNS queries and prevent upstream eavesdropping.

- WAN Ping Response Default: Routers are configured to ignore ICMP ping requests from public WAN interfaces by default.

- Session Timeout Rules: Inactive Web GUI management sessions automatically terminate after 10 minutes of idle time.

- Secure Boot Validation: Hardware firmware incorporates cryptographic secure boot checks to block tampered bootloaders.

- IoT Network Isolation: Dedicated IoT SSIDs automatically restrict client devices from communicating with local PC/NAS shares.

- VPNSecurity Protocol Support: Built-in VPN server functions support WireGuard and OpenVPN protocols with RSA-4096 keys.

- Credential Export Warnings: Exported router configuration files containing passwords are encrypted with user-defined keys.

- MAC Address Randomization Support: Routers support seamless connectivity for client devices utilizing randomized private MACs.

- Cloud Authentication Standard: Cloud management accounts enforce Mandatory Multi-Factor Authentication (MFA) for administrative staff.

- GDPR Data Erasure Requests: Processing an account deletion request permanently wipes historical cloud router logs within 30 days.

- SPI Firewall Default: Stateful Packet Inspection (SPI) firewalls are permanently active across all incoming WAN interfaces.

- PORT Scanning Defense: Automated PORT scanning activity detected on WAN interfaces triggers temporary source IP blocking.

- Malicious Domain Blocking: Integrated web filters cross-reference threat intelligence databases to block phishing URLs real-time.

- Encrypted Management Traffic: All communications between local hardware and cloud controllers use TLS 1.3 encryption.

- Guest Isolation Strict Mode: Guest clients are blocked from accessing physical router IP administrative interfaces (Port 80/443).

- Regulatory Compliance Standards: Hardware meets FCC Part 15, CE, and RoHS standards for RF emissions and environmental safety.

#### Section 5: Environmental & Compliance (Policies 181–200)

- Energy Efficiency Standards: Wi-Fi devices comply with ENERGY STAR standards, featuring automated low-power night modes.

- RoHS Materials Restriction: Hardware components are manufactured strictly without lead, mercury, cadmium, or PBB flame retardants.

- Recyclable Packaging Mandate: Packaging materials must consist of at least 80% recycled paper fibers with soy-based inks.

- E-Waste Takeback Support: Free prepaid mailer bags are provided upon request to support responsible hardware disposal.

- Thermal Dissipation Design: Passive cooling designs avoid noisy fans, relying on internal aluminum heat sinks and airflow vents.

- Conflict-Free Mineral Policy: Tantalum, tin, tungsten, and gold used in circuit boards are sourced from verified conflict-free mines.

- RF Exposure Compliance: Operating distances must maintain a minimum 20 cm (8 inches) body separation from radiating antenna arrays.

- Acoustic Noise Limits: Operating hardware noise emissions must remain below 20 dBA under peak processing operations.

- Overvoltage Line Protection: Ethernet ports feature 4KV surge isolation to protect against indirect atmospheric lightning transients.

- Plastics Marking Standard: All plastic housing parts are stamped with ISO plastic recycling codes for post-consumer sorting.

- Hazardous Chemical Logging: Safety Data Sheets (SDS) for internal battery backup cells are accessible via support portals.

- WEEE Directive Compliance: Products sold in European regions feature the crossed-out wheelie bin symbol denoting mandatory e-waste separation.

- Power Adapter Efficiency: Included AC/DC power adapters meet Level VI Department of Energy efficiency requirements.

- Packaging Plastic Reduction: Single-use plastic wraps around cables and power bricks are replaced with biodegradable paper ties.

- Operating Humidity Range: Hardware operates reliably in non-condensing relative humidity conditions between 10% and 90%.

- Vibration Resilience Testing: Packaging and hardware pass ISTA 2A drop and shock vibration transportation testing standards.

- Carbon Footprint Disclosures: Product lifecycle carbon footprint assessment metrics are published annually in sustainability reports.

- Refurbishment Quality Standard: Factory refurbished hardware undergoes 100% full functional testing equal to new production units.

- End-of-Life Notification Notice: Customers receive 6 months advance notice before hardware reaches official End-of-Support (EoS) status.

- Global Regulatory Compliance: Devices maintain certified regional compliance marks including FCC, IC, CE, UKCA, and TELEC.